SMT - SECS/GEM

User manual

1.0.0.6

2022/05/17

大量科技股份有限公司 TALIANG TECHNOLOGY

桃園市八德區友聯街49號 No. 49, Youlian St., Bade Dist., Taoyuan City 334009, Taiwan

TEL : (03)3686368 / FAX : (03)3685811

目錄

Install

SECS/GEM Driver & Dongle Install 6

User Interface

SECS/GEM Interface - State7

SECS/GEM Interface - Default SubState10

SECS/GEM Interface - Manual SV,DV,EC11

SECS/GEM Interface - Manual Alarm14

SECS/GEM Interface - Manual Event15

SECS/GEM Interface - Manual Spooling16

SECS/GEM Interface - Manual Terminal17

SECS/GEM Interface - Manual PP18

Config Interface 19

SYSTEM ID

SYSTEM SV 20

SYSTEM DV 22

SYSTEM EC 23

SYSTEM EVENT 25

EQUIPMENT ID

EQUIPMENT SV 26

EQUIPMENT DV 27

EQUIPMENT EVENT 28

EQUIPMENT ALARM29

SECS/GEM REPORT

EVENT REPORT LINK 36

SECS-II Message LIST

Message List (H -> E) 38

Message List (E -> H) 39

SECS-II Message Example

Stream 1 : Equipment Status

S1F1 : Are You There Request (H -> E) 40

S1F2 : On Line Data (E -> H) 40

S1F3 : Selected Equipment Status Request (H -> E) 41

S1F4 : Selected Equipment Status Data (E -> H) 41

S1F11 : Status Variable Name List Request (H -> E) 42

S1F12 : Status Variable name List Reply (E -> H) 42

S1F13 : Establish Communications Request (H -> E) 43

S1F14 : Establish Communications Request Acknowledge (E -> H) 43

S1F15 : Request Off-Line (H -> E) 44

S1F16 : Off-Line Acknowledge (E -> H) 44

S1F17 : Request On-Line (H -> E) 45

S1F18 : On-Line Acknowledge (E -> H) 45

Stream 2 : Equipment Control and Diagnostics

S2F13 : Equipment Constant Request (H -> E) 46

S2F14 : Equipment Constant Data (E -> H) 46

S2F15 : New Equipment Constant Send (H -> E) 47

S2F16 : New Equipment Constant Acknowledge (E -> H) 47

S2F17 : Date and Time Request (H -> E) 48

S2F18 : Date and Time Data (E -> H) 48

S2F23 : Trace Initialize Send (H -> E) 49

S2F24 : Trace Initialize Acknowledge (E -> H) 49

S2F25 : Loop Back Diagnosis Request (H -> E) 50

S2F26 : Loop Back Diagnosis Reply (E -> H) 50

S2F29 : Equipment Constant Name List Request (H -> E) 51

S2F30 : Equipment Constant Name List Reply (E -> H) 51

S2F31 : Date and Time Send (H -> E) 52

S2F32 : Date and Time Acknowledge (E -> H) 52

S2F33 : Define Report (H -> E) 53

S2F34 : Define-Report Acknowledge (E -> H) 53

S2F35 : Link Event Report (H -> E) 54

S2F36 : Link Event Report Acknowledge (E -> H) 54

S2F37 : Enable/Disable Event Report (H -> E) 55

S2F38 : Enable/Disable Event Report Acknowledge (E -> H) 55

S2F41 : Remote Command Send (H -> E) 56

S2F42 : Remote Command Acknowledge (E -> H) 57

Stream 5 : Alarm Handling

S5F1 : Alarm Report Send (E -> H) 58

S5F2 : Alarm Report Acknowledge (H -> E) 58

S5F3 : Enable/Disable Alarm Send (H -> E) 59

S5F4 : Enable/Disable Alarm Acknowledge (E -> H) 59

S5F5 : List Alarms Request (H -> E) 60

S5F6 : List Alarm Data (E -> H) 60

S5F7 : List Enabled Alarms Request (H -> E) 61

S5F8 : List Enabled Alarm Data (E -> H) 61

Stream 6 : Data Collection

S6F1 : Trace Data Send (E -> H) 62

S6F2 : Trace Data Acknowledge (H -> E) 62

S6F11 : Event Report Send (E -> H) 63

S6F12 : Event Report Acknowledge (H -> E) 63

S6F15 : Event Report Request (H -> E) 64

S6F16 : Event Report Data (E -> H) 64

S6F19 : Individual Report Request (H -> E) 65

S6F20 : Individual Report Data (E -> H) 65

Stream 7 : Process Program Managed

S7F1 : Process Program Load Inquire (H -> E) 66

S7F2 : Process Program Load Grant (E -> H) 66

S7F5 : Process Program Request (H -> E) 67

S7F6 : Process Program Data (E -> H) 67

S7F19 : Current Process Program Dir. Request (H -> E) 68

S7F20 : List Process Program Data (E -> H) 68

S7F25 : Formatted Process Program Request (H -> E) 69

S7F26 : Formatted Process Program Data (E -> H) 69

Stream 9 : System Errors

S9F7 : llegal Data (E -> H) 70

S9F9 : Trace Data Send (E -> H) 70

Stream 10 : Terminal Services

S10F1 : Terminal Request (E -> H) 71

S10F2 : Terminal Request Acknowledge (H -> E) 71

S10F3 : Terminal Display (H -> E) 72

S10F4 : Terminal Display Acknowledge (E -> H) 72

SMT-SECS/GEM Infomation

PPBODY 73

Magazine MoveIn Log75

Boat MoveIn Log76

PassBoat MoveOut Log77

NGBoat MoveOut Log78

EmptyBoat Release Log79

Pass1Magazine MoveIn Log80

Pass2Magazine MoveIn Log81

NGMagazine MoveIn Log82

EmptyMagazine MoveIn Log83

Pass1Magazine MoveOut Log84

Pass2Magazine MoveOut Log85

NGMagazine MoveIn Log86

Boat Assing NGBoat Log87

UI Lock Command Log88

Equipment Operational Command Log90

Equipment EndLot Command Log92

Equipment StartRun Check Log93

SECS/GEM Driver & Dongle Install

| STEP1 : Install HASPUserSetup.exe |  |
| --- | --- |
| STEP2 : Insert SECS/GEM Dongle to Computer’s USB port |  |

※If the installation fails or the USB cannot be read, the message box will pop up as shown below.

SECS/GEM Interface - State

SECS Connection state

Communicating state

Enable/Disable Comm

Display of current control status

Status toggle field

Messagebox

GEM Initial result show

GEM Clock

GEM MODEL Name

GEM Software Version

QuickGEM Software Version

EQ Status show

Spooling State show

Are you there command

Last command result

EnableComm : Start Communication

------------------------------------------------------------------------------------

DisableComm : Stop Communication

------------------------------------------------------------------------------------

On Line Request : Send the request to connect with the remote On Line

------------------------------------------------------------------------------------

Off Line : Send the request to disconnect with the remote On Line

On Line Remote : Switch to remote control status

------------------------------------------------------------------------------------

On Line Local : Switch to local control status

SECS/GEM Interface – Default Substate

Switch EC:8, EC:49, EC:50, EC:51 ‘s sub state, See [System EC page (p.23) ] for details

SECS/GEM Interface – Manual SV,DV,EC

Update EC : Update specified Equipment Constant(EC) value

Update SV(DV) : Update specified Status Variable(SV) and Data Value(DV) value

------------------------------------------------------------------------------------

GetEC : Load specified Equipment Constant value

GetSV(DV) : Load specified Status Variable(SV) and Data Value(DV) value

SECS/GEM Interface – Alarm

AlarmReportSend : Based on specified Alarm ID send alarm message

------------------------------------------------------------------------------------

S5 WBit <EC:21> : See [System EC page (p.23) ] for details

SECS/GEM Interface – Event

EventReportSend : Based on specified Event ID send event message

------------------------------------------------------------------------------------

S6 WBit <EC:22> : See [System EC page (p.23) ] for details

SECS/GEM Interface – Spooling

Display SV:53, SV:54, SV:55, SV:57 current value, See [System SV page (p.20) ] for details

Display EC:52current value, See [System EC page (p.23) ] for details

Open/Close EC66, EC67 enable state, See [System EC page (p.23) ] for details

SECS/GEM Interface – Terminal

Msg Recognition Event Send: Send terminal message

------------------------------------------------------------------------------------

Terminal Message Send: Send terminal message

------------------------------------------------------------------------------------

S10 WBit <EC:23> : See [System EC page (p.23) ] for details

SECS/GEM Interface – PP

Function specified by excute CmdID

Config Interface

Role : Set the machine as the active side (Host) or the passive side (Equipment)

T3 : Set the time for the sender to wait for the receiver to return a message after sending the data message

T5 : After the TCP/IP connection is set to fail, the active side needs to wait for T5 time before reconnecting

T6 : Set the time for the sender to wait for the receiver to return a message after sending a control message

T7 : Set the time to wait to enter Selected Stats after entering Connected Stats

T8 : Set the maximum time interval allowed between characters in the message when the receiver receives the message

Local IP & Port : Set local IP & Port

Remote IP & Port : Set remote IP & Port

Device ID : Set Device ID

Linktest Period : Set the interval for sending test messages periodically (limited by T6)

System SV

System DV

System EC

System Event

Equpiment SV

| ID | TYPE | Lenght | NAME | Definition |
| --- | --- | --- | --- | --- |
| 1001 | ASCII | 1000 | MagazineIDCheck | 入料端MagazineID確認 |
| 1002 | ASCII | 1000 | BoatBarcodeCheck | 入料端BoatID確認 |
| 1003 | ASCII | 1000 | Pass1MagazineIDCheck | 出料端Pass1MagazineID確認 |
| 1004 | ASCII | 1000 | Pass2MagazineIDCheck | 出料端Pass2MagazineID確認 |
| 1005 | ASCII | 1000 | NGMagazineIDCheck | 出料端NGMagazineID確認 |
| 1006 | ASCII | 1000 | EmptyMagazineIDCheck | 出料端EmptyMagazineID確認 |
| 1007 | ASCII | 1000 | BoatAssingNGBoatBarcodeCheck | 出料端 空Boat回流登記NGBoat確認 |
| 1008 | ASCII | 1000 | UILockCommand | 遠端切換UI鎖定/解鎖狀態指令 |
| 1009 | ASCII | 1000 | EquipmentOperationalCommand | 遠端切換設備啟動/停止狀態指令 |
| 1010 | ASCII | 1000 | EquipmentEndLotCommand | 遠端控制設備結批指令 |
| 1011 | ASCII | 1000 | RemoteActionResult | 遠端指令執行結果 |
| 1012 | ASCII | 1000 | EmptyBoatReleaseCheck | 空Boat Release狀態 確認 |
| 1013 | ASCII | 1000 | EquipmentStartRunCheck | 設備啟動狀態確認 |

Equpiment DV

| ID | TYPE | Lenght | NAME | Definition |
| --- | --- | --- | --- | --- |
| 4001 | ASCII | 1000 | LotID | Lot ID |
| 4002 | ASCII | 1000 | MagazineID | 入料端 MagazineID |
| 4003 | ASCII | 1000 | Pass1MagazineID | 出料端 Pass1MagazineID |
| 4004 | ASCII | 1000 | Pass2MagazineID | 出料端 Pass2MagazineID |
| 4005 | ASCII | 1000 | NGMagazineID | 出料端 NGMagazineID |
| 4006 | ASCII | 1000 | EmptyMagazineID | 出料端 EmptyMagazineID |
| 8001 | ASCII | 1000 | BoatBarcode | 入料端Boat條碼 |
| 8002 | ASCII | 1000 | PassBoatBarcode | 出料端PassBoat條碼 |
| 8003 | ASCII | 1000 | NGBoatBarcode | 出料端NGBoat條碼 |
| 8004 | ASCII | 1000 | EmptyBoatBarcode | 出料端 空Boat條碼 |
| 8005 | ASCII | 1000 | BoatAssingNGBoatBarcode | 出料端 空Boat回流登記NGBoat條碼 |
| 8006 | ASCII | 1000 | RecipeName | 配方名稱 |
| 8007 | ASCII | 1000 | Pass1MagazineUnitCount | 出料端 Pass1MagazineID Unit總數 |
| 8008 | ASCII | 1000 | Pass2MagazineUnitCount | 出料端 Pass2MagazineID Unit總數 |
| 8009 | ASCII | 1000 | NGMagazineUnitCount | 出料端 NGMagazineID Unit總數 |

Equpiment Event

| ID | NAME | Definition |
| --- | --- | --- |
| 101 | MagazineMoveInEvent | 入料端Magazine MoveIn事件 |
| 102 | BoatMoveInEvent | 入料端Boat MoveIn事件 |
| 103 | PassBoatMoveOutEvent | 出料端PassBoat MoveOut事件 |
| 104 | NGBoatMoveOutEvent | 出料端NGBoat MoveOut事件 |
| 105 | EmptyBoatMoveOutEvent | 出料端EmptyBoat MoveOut事件 |
| 106 | Pass1MagazineMoveInEvent | 出料端 Pass1Magazine MoveIn事件 |
| 107 | Pass2MagazineMoveInEvent | 出料端 Pass2Magazine MoveIn事件 |
| 108 | NGMagazineMoveInEvent | 出料端 NGMagazine MoveIn事件 |
| 109 | EmptyMagazineMoveInEvent | 出料端EmptyMagazine MoveIn事件 |
| 110 | Pass1MagazineMoveOutEvent | 出料端 Pass1Magazine MoveOut事件 |
| 111 | Pass2MagazineMoveOutEvent | 出料端 Pass2Magazine MoveOut事件 |
| 112 | NGMagazineMoveOutEvent | 出料端 NGMagazine MoveOut事件 |
| 113 | BoatAssingNGBoatEvent | 出料端 空Boat回流登記NGBoat事件 |
| 114 | RemoteActionResultEvent | 遠端指令執行結果事件 |
| 115 | EquipmentStartRunEvent | 設備啟動狀態事件 |

Equpiment Alarm - Loader

| ID | Name | Definition |
| --- | --- | --- |
| 10001 | Loader - Rail door inspection abnormal | Loader - 門檢異常 |
| 10002 | Loader - The emergency stop was pressed | Loader - 緊急停止被按下 |
| 10003 | Loader - Air Pressure Low(X02) | Loader - 氣壓不足(X02) |
| 10004 | Loader - Jaws cylinder abnormal (X03) | Loader - 夾爪氣缸動檢(X03)-異常 |
| 10005 | Loader - Jaws cylinder abnormal (X04) | Loader - 夾爪氣缸原點檢(X04)-異常 |
| 10006 | Loader - Jaws Right side abnormal (X05) | Loader - 料匣感知-R(X05)-異常 |
| 10007 | Loader - Jaws Left side abnormal (X06) | Loader - 料匣感知-L(X06)-異常 |
| 10008 | Loader - Inlet area Z position wrong(X07) | Loader - 入料區-夾匣位置檢(X07)-異常 |
| 10009 | Loader - Outlet area Z position wrong(X08) | Loader - 出料區-夾匣位置檢(X08)-異常 |
| 10010 | Loader - Boat stuck(X09) | Loader - Boat盤推移防撞檢(X09)-異常 |
| 10011 | Loader - Inlet area arrive sensor error(X10) | Loader - 入料區-料匣到位檢(X10)-異常 |
| 10012 | Loader - Inlet CV motor error(X11) | Loader - 入料區-料匣皮帶馬達(X11)-異常 |
| 10013 | Loader - Outlet area arrive sensor error(X12) | Loader - 出料區-料匣到位檢(X12)-異常 |
| 10014 | Loader - Outlet CV motor error(X13) | Loader - 出料區-料匣皮帶馬達(X13)-異常 |
| 10015 | Loader - Boat fall down(X14) | Loader - Boat盤入料檢(X14)-異常 |
| 10016 | Loader - Boat fall down(X18) | Loader - Boat盤出料檢(X18)-異常 |
| 10017 | Loader - Track side CV motor error(X19) | Loader - Boat盤皮帶馬達(X19)-異常 |
| 10018 | Loader - Inlet area need Magazine(X21) | Loader - 入料區-料匣缺料檢知(X21)-異常 |
| 10019 | Loader - Outlet area Full(X22) | Loader - 出料區-料匣滿料檢知(X22)-異常 |
| 10020 | Loader - X position wrong can not move Z(X23) | Loader - Boat盤X軸安全位置檢(X23)-異常 |
| 10021 | Loader - Obstacles above the track(X24) | Loader - Boat盤晶片平整檢1(X24)-異常 |
| 10022 | Loader - Obstacles above the track(X25) | Loader - Boat盤晶片平整檢2(X25)-異常 |
| 10023 | Loader - Magazine Barcode Error | Loader - 料匣-BarCode-異常 |
| 10024 | Loader - Boat Barcode Error | Loader - Boat-BarCode-異常 |
| 10025 | Loader - Sensor X15 is on | Loader - Boat盤位置檢1(X15)-異常 |
| 10026 | Loader - Sensor X16 is on | Loader - Boat盤位置檢2(X16)-異常 |
| 10027 | Loader - Sensor X17 is on | Loader - Boat盤位置檢3(X17)-異常 |
| 10028 | Loader - Boat Enter Error | Loader - Boat入料異常 |
| 10029 | Loader - Boat error not manually removed | Loader - Boat盤異常未手動移出。 |

Equpiment Alarm - Sorter

| ID | Name | Definition |
| --- | --- | --- |
| 10201 | Sorter - Rail door inspection abnormal | Sorter - 門檢異常 |
| 10202 | Sorter - The emergency stop was pressed. | Sorter - 緊急停止被按下 |
| 10203 | Sorter - Air Pressure Low(X02) | Sorter - 氣壓不足(X02) |
| 10204 | Sorter - Vacuum Low(X03) | Sorter - 真空不足(X03) |
| 10205 | Sorter - Turrets does not at upper(X04) | Sorter - 吸筆不在上位(X04) |
| 10206 | Sorter - Unit missing(X05) | Sorter - 材料檢異常(X05) |
| 10207 | Sorter - Vacuum Low(X06) | Sorter - 真空檢異常(X06) |
| 10208 | Sorter - Door stop cylinder abnormal (X07) | Sorter - ST區Boat盤門擋汽缸動檢(X07)-異常 |
| 10209 | Sorter - Door stop cylinder abnormal (X08) | Sorter - ST區Boat盤門擋汽缸原點檢(X08)-異常 |
| 10210 | Sorter - Aside cylinder abnormal (X09) | Sorter - ST區Boat盤靠邊汽缸動檢(X09)-異常 |
| 10211 | Sorter - Aside cylinder abnormal (X10) | Sorter - ST區Boat盤靠邊汽缸原點檢(X10)-異常 |
| 10212 | Sorter - Door stop cylinder abnormal (X11) | Sorter - OK區Boat盤門擋汽缸動檢(X11)-異常 |
| 10213 | Sorter - Door stop cylinder abnormal (X12) | Sorter - OK區Boat盤門擋汽缸原點檢(X12)-異常 |
| 10214 | Sorter - Aside cylinder abnormal (X13) | Sorter - OK區Boat盤靠邊汽缸動檢(X13)-異常 |
| 10215 | Sorter - Aside cylinder abnormal (X14) | Sorter - OK區Boat盤靠邊汽缸原點檢(X14)-異常 |
| 10216 | Sorter - Door stop cylinder abnormal (X15) | Sorter - NG區Boat盤門擋汽缸動檢(X15)-異常 |
| 10217 | Sorter - Door stop cylinder abnormal (X16) | Sorter - NG區Boat盤門擋汽缸原點檢(X16)-異常 |
| 10218 | Sorter - Aside cylinder abnormal (X17) | Sorter - NG區Boat盤靠邊汽缸動檢(X17)-異常 |
| 10219 | Sorter - Aside cylinder abnormal (X18) | Sorter - NG區Boat盤靠邊汽缸原點檢(X18)-異常 |
| 10220 | Sorter - Boat fall down(X19) | Sorter - ST區Boat盤入料檢(X19)-異常 |
| 10221 | Sorter - Obstacles above the track(X21) | Sorter - ST區Boat盤晶片平整檢1(X21)-異常 |
| 10222 | Sorter - Obstacles above the track(X24) | Sorter - ST區Boat盤晶片平整檢2(X24)-異常 |
| 10223 | Sorter - Boat fall down(X26) | Sorter - ST區Boat盤出料檢(X26)-異常 |
| 10224 | Sorter - Boat fall down(X27) | Sorter - OK區Boat盤入料檢(X27)-異常 |
| 10225 | Sorter - Obstacles above the track(X29) | Sorter - OK區Boat盤晶片平整檢1(X29)-異常 |
| 10226 | Sorter - Obstacles above the track(X32) | Sorter - OK區Boat盤晶片平整檢2(X32)-異常 |
| 10227 | Sorter - Boat fall down(X34) | Sorter - OK區Boat盤出料檢(X34)-異常 |
| 10228 | Sorter - Boat fall down(X35) | Sorter - NG區Boat盤入料檢(X35)-異常 |
| 10229 | Sorter - Obstacles above the track(X37) | Sorter - NG區Boat盤晶片平整檢1(X37)-異常 |
| 10230 | Sorter - Obstacles above the track(X40) | Sorter - NG區Boat盤晶片平整檢2(X40)-異常 |
| 10231 | Sorter - Boat fall down(X42) | Sorter - NG區Boat盤出料檢(X42)-異常 |
| 10232 | Sorter - Outlet CV motor error(X43) | Sorter - ST區-Boat盤皮帶馬達(X43)-異常 |
| 10233 | Sorter - Outlet CV motor error(X44) | Sorter - OK區-Boat盤皮帶馬達(X44)-異常 |
| 10234 | Sorter - Outlet CV motor error(X45) | Sorter - NG區-Boat盤皮帶馬達(X45)-異常 |
| 10235 | Sorter - Illegal IP | Sorter - 非法IP |
| 10236 | Sorter - IP conflict | Sorter - IP衝突 |
| 10237 | Sorter - Port error | Sorter - Port 錯誤 |
| 10238 | Sorter - Servere establishment failed | Sorter - Servere建立失敗 |
| 10239 | Sorter - Client connection failed | Sorter - Client連線失敗 |
| 10240 | Sorter - Pen Side Moter X Error | Sorter - 吸筆X軸馬達異常 |
| 10241 | Sorter - Pen Side Moter Y Error | Sorter - 吸筆Y軸馬達異常 |
| 10242 | Sorter - Pen Side Moter Z Error | Sorter - 吸筆Z軸馬達異常 |
| 10243 | Sorter - Under side Moter T Error | Sorter - 底盤Y向馬達異常 |
| 10244 | Sorter - Inbound and outbound detection detected objects | Sorter - 入出站檢偵測到物體 |
| 10245 | Sorter - Abnormal height of pen | Sorter - 吸筆高度異常 |
| 10246 | Sorter - Servo motor not Servo On | Sorter - CV馬達異常 |
| 10247 | Sorter - Servere establishment failed | Sorter - 伺服馬達未Servo On |
| 10248 | Sorter - Call Unloader go to swap area fail | Sorter - 呼叫 Unloader 去交換區 失敗 |
| 10249 | Sorter - Execute UnloadBoat fail | Sorter - 執行 UnLoadBoat 失敗 |
| 10250 | Sorter - Find warping in OK area | Sorter - OK區發現翹料 |
| 10251 | Sorter - Find warping in NG area | Sorter - NG區發現翹料 |
| 10252 | Sorter - Find warping in ST area | Sorter - ST區發現翹料 |

Equpiment Alarm - Unloader

| ID | Name | Definition |
| --- | --- | --- |
| 10401 | Unloader - Rail door inspection abnormal | Unloader - 門檢異常 |
| 10402 | Unloader - The emergency stop was pressed. | Unloader - 緊急停止被按下 |
| 10403 | Unloader - Air Pressure Low(X02) | Unloader - 氣壓不足(X02) |
| 10404 | Unloader - Jaws cylinder abnormal (X03) | Unloader - 夾爪氣缸動檢(X03)-異常 |
| 10405 | Unloader - Jaws cylinder abnormal (X04) | Unloader - 夾爪氣缸原點檢(X04)-異常 |
| 10406 | Unloader - Jaws cylinder X side abnormal (X05) | Unloader - 夾爪X軸氣缸動檢(X05)-異常 |
| 10407 | Unloader - Jaws cylinder X side abnormal (X06) | Unloader - 夾爪X軸氣缸原點檢(X06)-異常 |
| 10408 | Unloader - Jaws cylinder Z side abnormal (X07) | Unloader - 夾爪Z軸氣缸動檢(X07)-異常 |
| 10409 | Unloader - Jaws cylinder Z side abnormal (X08) | Unloader - 夾爪Z軸氣缸原點檢(X08)-異常 |
| 10410 | Unloader - Move stage cylinder abnormal (X09) | Unloader - 移載汽缸動檢(X09)-異常 |
| 10411 | Unloader - Move stage cylinder abnormal (X10) | Unloader - 移載汽缸原點檢(X10)-異常 |
| 10412 | Unloader - Boat fall down(X11) | Unloader - Boat盤入料檢(X11)-異常 |
| 10413 | Unloader - Boat fall down(X15) | Unloader - Boat盤出料檢(X15)-異常 |
| 10414 | Unloader - Track side CV motor error(X16) | Unloader - Boat盤皮帶馬達(X16)-異常 |
| 10415 | Unloader - Missing Magazine 1 | Unloader - 料匣1到位檢(X17)-異常 |
| 10416 | Unloader - Missing Magazine 2 | Unloader - 料匣2到位檢(X18)-異常 |
| 10417 | Unloader - Missing Magazine 3 | Unloader - 料匣3到位檢(X19)-異常 |
| 10418 | Unloader - Missing Magazine 4 | Unloader - 料匣4到位檢(X20)-異常 |
| 10419 | Unloader - Obstacles above the track(X22) | Unloader - Boat盤晶片平整檢1(X22)-異常 |
| 10420 | Unloader - Obstacles above the track(X23) | Unloader - Boat盤晶片平整檢2(X23)-異常 |
| 10421 | Unloader - Magazine 1 Missing Boat | Unloader - 料匣1-Boat盤在籍檢(X27)-異常 |
| 10422 | Unloader - Magazine 2 Missing Boat | Unloader - 料匣2-Boat盤在籍檢((X28)-異常 |
| 10423 | Unloader - Magazine 3 Missing Boat | Unloader - 料匣3-Boat盤在籍檢((X29)-異常 |
| 10424 | Unloader - Magazine 4 Missing Boat | Unloader - 料匣4-Boat盤在籍檢((X30)-異常 |
| 10425 | Unloader - Magazine Barcode Error | Unloader - 料匣-BarCode-異常 |
| 10426 | Unloader - Boat Barcode Error | Unloader - Boat-BarCode-異常 |
| 10427 | Unloader - Sensor X12 is on | Unloader - Boat盤位置檢1(X12)-異常 |
| 10428 | Unloader - Sensor X13 is on | Unloader - Boat盤位置檢2(X13)-異常 |
| 10429 | Unloader - Sensor X14 is on | Unloader - Boat盤位置檢3(X14)-異常 |
| 10430 | Unloader - Boat Enter Error | Unloader - Boat入料異常 |
| 10431 | Unloader - Illegal IP | Unloader - 非法IP |
| 10432 | Unloader - IP conflict | Unloader - IP衝突 |
| 10433 | Unloader - Port error | Unloader - Port 錯誤 |
| 10434 | Unloader - Servere establishment failed | Unloader - Servere建立失敗 |
| 10435 | Unloader - Client connection failed | Unloader - Client連線失敗 |
| 10436 | Unloader - Pen Side Moter X Error | Unloader - Y軸馬達異常 |
| 10437 | Unloader - Pen Side Moter Z Error | Unloader - 乘載板Z軸馬達異常 |
| 10438 | Unloader - The in-out material inspection detects an object or the cylinder is not in place | Unloader - 入出料檢偵測到物體或汽缸未到位 |
| 10439 | Unloader - Unable to set motor parameters | Unloader - 無法設定馬達參數 |
| 10440 | Unloader - The number of MAG layers set abnormally | Unloader - MAG層數設定異常 |
| 10441 | Unloader - Find warping | Unloader - 發現翹料 |

Equpiment Alarm - AOI

| ID | Name | Definition |
| --- | --- | --- |
| 10601 | AOI - Rail door inspection abnormal | AOI - 門檢異常 |
| 10602 | AOI - The emergency stop was pressed | AOI - 緊急停止被按下 |
| 10603 | AOI - Air Pressure Low(X02) | AOI - 氣壓不足(X02) |
| 10604 | AOI - Clip cylinder abnormal (X05) | AOI - 夾板氣缸(A)動檢(X05)-異常 |
| 10605 | AOI - Clip cylinder abnormal (X06) | AOI - 夾板氣缸(A)原點檢(X06)-異常 |
| 10606 | AOI - Clip cylinder abnormal (X07) | AOI - 夾板氣缸(B)動檢(X07)-異常 |
| 10607 | AOI - Clip cylinder abnormal (X08) | AOI - 夾板氣缸(B)原點檢(X08)-異常 |
| 10608 | AOI - Aside cylinder abnormal (X13) | AOI - 靠邊氣缸動檢(X13)-異常 |
| 10609 | AOI - Aside cylinder abnormal (X14) | AOI - 靠邊氣缸原點檢(X14)-異常 |
| 10610 | AOI - Boat fall down(X15) | AOI - Boat盤入口下垂檢(X15)-異常 |
| 10611 | AOI - Boat fall down(X16) | AOI - Boat盤出口下垂檢(X16)-異常 |
| 10612 | AOI - Obstacles above the track(X17) | AOI - Boat盤晶片平整檢1(X17)-異常 |
| 10613 | AOI - Obstacles above the track(X18) | AOI - Boat盤晶片平整檢2(X18)-異常 |
| 10614 | AOI - Track side CV motor error(X22) | AOI - Boat盤皮帶馬達(X22)-異常 |
| 10615 | AOI - Door stop cylinder abnormal (X23) | AOI - 門檔氣缸動檢-異常(X23) |
| 10616 | AOI - Door stop cylinder abnormal (X24) | AOI - 門檔氣缸原點檢-異常(x24) |
| 10617 | AOI - NG counter arrive setting | AOI - NG數量達設定值 |
| 10701 | AOI - Clamp A,B Initial retrack TimeOut | AOI - Clmap A 或 Clamp B 回原點逾時 |
| 10702 | AOI - Z Axis Move TimeOut | AOI - Z軸歸零時移動逾時 |
| 10703 | AOI - Z Axis Homing TimeOut | AOI - Z軸歸零逾時 |
| 10704 | AOI - Initial Stopper Rising TimeOut | AOI - 初始化Stopper頂昇逾時 |
| 10705 | AOI - X Axis Move 10mm TimeOut | AOI - X軸移動10mm逾時 |
| 10706 | AOI - Y Axis Move 10mm TimeOut | AOI - Y軸移動10mm逾時 |
| 10707 | AOI - X Axis Homing TimeOut | AOI - X軸歸零逾時 |
| 10708 | AOI - Y Axis Homing TimeOut | AOI - Y軸歸零逾時 |
| 10801 | AOI - Stopper Rising TimeOut | AOI - Stopper升起逾時 |
| 10802 | AOI - Load Product TimeOut | AOI - 產品在流道上入料逾時 |
| 10803 | AOI - Load Product Arrial Stopper TimeOut | AOI - 產品到位逾時 |
| 10804 | AOI - SidePusth stretch out TimeOut | AOI - 側推伸出失敗 |
| 10805 | AOI - Clamp A or Clamp B stretch out TimeOut | AOI - 入料時 Clmap A或Clamp B頂升逾時 |
| 10806 | AOI - SidePush retract out TimeOut | AOI - 側推縮回逾時 |
| 10807 | AOI - Clamp A or Clamp B retrack out TimeOut | AOI - 入料時 Clmap A或Clamp B下降逾時 |
| 10808 | AOI - Stopper retrack TimeOut | AOI - Stopper縮回逾時 |
| 10809 | AOI - Unload Product TimeOut | AOI - 出料逾時(產品未離開流道Sensor) |
| 10810 | AOI - Stopper Rising TimeOut After Unload Product | AOI - 產品出料後 Stoppe頂昇逾時 |
| 10901 | AOI - AOI Move to Start Pos. of the row TimeOut | AOI - AOI移動至單列起跑點逾時 |
| 10902 | AOI - AOI Move to Row End Pos. of the row TimeOut | AOI - AOI移動至單列結束點逾時 |

Event Report Link

Equipment event (p.28) will be transmitted with RTPID and SV, DV, EC values ​​will be transmitted together, as shown below.

| EventID | RPTID | SVID, DVID, ECID |
| --- | --- | --- |
| MagazineInEvent(101) | 101 | MagazineID(4002) |
| BoatMoveInEvent(102) | 102 | BoatBarcode(8001) |
| PassBoatMoveOutEvent(103) | 103 | PassBoatBarcode(8002) |
| NGBoatMoveOutEvent(104) | 104 | NGBoatBarcode(8003) |
| EmptyBoatReleaseEvent(105) | 105 | EmptyBoatBarcode(8004) |
| Pass1MagazineMoveInEvent(106) | 106 | Pass1MagazineID(4003) |
| Pass2MagazineMoveInEvent(107) | 107 | Pass2MagazineID(4004) |
| NGMagazineMoveInEvent(108) | 108 | NGMagazineID(4005) |
| EmptyMagazineMoveInEvent(109) | 109 | EmptyMagazineID(4006) |
| Pass1MagazineMoveOutEvent(110) | 110 | LotID(4001) Pass1MagazineID(4003) Pass1MagazineUnitCount(8007) |
| Pass2MagazineMoveOutEvent(111) | 111 | LotID(4001) Pass2MagazineID(4004) Pass2MagazineUnitCount(8008) |
| NGMagazineMoveOutEvent(112) | 112 | LotID(4001) NGMagazineID(4005) NGMagazineUnitCount(8009) |
| BoatAssingNGBoatEvent(113) | 113 | LotID(4001) BoatAssingNGBoatBarcode(8005) |
| RemoteActionResultEvent(114) | 114 | RemoteActionResult(1011) |
| EquipmentStartRunEvent(115) | 115 | RecipeName(8006) |

SECS-II Message List (H -> E)

SECS-II Message List (E -> H)

SECS-II Message Example

(Host perspective)

S1F1 : Are You There  (H -> E)

Description :

The host may send this message to the EQ at any time. EQ sends this message periodically as a “heartbeat” to determine if the SECS link is alive.

S1F2 : On-Line Data  (E -> H)

Description :

EQ reports its Model Number and Software Revision to Host

.

S1F3 : Selected Status Request  (H -> E)

Description :

A request to EQ to report selected values of its status.

S1F4 : Selected Status Data  (E -> H)

Description :

The equipment reports the value of each SVID requested in the order requested.

S1F11 : Status Variable NamelistRequest  (H -> E)

Description :

A request to the equipment to identify certain status variables.

S1F12 : Status Variable Namelist Reply  (E -> H)

Description :

EQ reports to the Host the name and units of the requested SVs.

S1F13 : Establish Communication Request  (H -> E)

Description:

Either end of the link may send S1F13 as the first message to establish connection of the link. Under EQ’s initialization logic, EQ will always attempt to establish the link. If no Host acceptance reply is received. EQ sends this message periodically.

S1F14 : Establish Communication Request Acknowledge  (E -> H)

Description:

Accept or deny Establish Communication Request.

S1F15 : Request Off-Line  (H -> E)

Description:

Host requests EQ to go Off-Line

S1F16 : Off-Line Acknowledge  (E -> H)

Description:

EQ control state transits to Host Off-Line and sends this message in response to S1F15 primary.

S1F17 : Request On-Line  (H -> E)

Description:

Host requests EQ to go On-Line

S1F18 : On-Line Acknowledge  (E -> H)

Description:

EQ responds to S1F17 primary in an attempt to go On-Line. Data item ONLACK indicates the success of failure of the attempt.

S2F13 : Equipment Constant Request  (H -> E)

Description:

Constants and other values that are changed infrequently can be obtained using this message.

S2F14 : Equipment Constant Data  (E -> H)

Description:

EQ returns constant data in the order requested in S2F13.

S2F15 : New Equipment Constant Send  (H -> E)

Description:

Change one or more equipment constants.

S2F16 : Equipment Constant Send Acknowledge  (E -> H)

Description:

Change one or more equipment constants.

S2F17 : Date and Time Request  (H -> E)

Description:

Host sends this message at any time to determine the date and time base EQ is currently using.

S2F18 : Date and Time Data  (E -> H)

Description:

Host sends this message at any time to determine the date and time base EQ is currently using.

S2F23 : Trace Initialize Send  (H -> E)

Description:

Trace initial send to device to enable/disable trace initial data. The trace data is returned on (S6F1) and is related to the original request by the TRID.

S2F24 : Trace Initialize Acknowledge  (E -> H)

Description:

Acknowledge or error for Trace Initialize Send(S2F23)

S2F25 : Diagnostic Loopback Request  (H -> E)

Description:

Host sends an arbitrary binary string to test operation of the link.

S2F26 Diagnostic Loopback Data  (E -> H)

Description:

EQ “echoes” back the binary data from Host’s S2F25.

S2F29 Equipment Constant Namelist Request  (H -> E)

Description:

This function allows the host to retrieve basic information about what equipment constants are available in the equipment.

S2F30 : Equipment Constant Namelist Reply  (E -> H)

Description:

Data Response for Equipment Constant Namelist Request(S2F29).

S2F31 : Date and Time Send  (H -> E)

Description:

Useful to synchronize the equipment time with the host time base.

S2F32 Date and Time Acknowledge  (E -> H)

Description:

Acknowledge the receipt of time and date.

S2F33 Define Report  (H -> E)

Description:

The purpose of this message is for the host to define a group of reports for the equipment. The event report will be sent via S6F11 when the event occurred.

S2F34 Define Report Acknowledge  (E -> H)

Description:

Acknowledge or error if an error condition is detected the entire message is rejected(i.e., partial changes are not allowed)

S2F35 : Link Event Report  (H -> E)

Description:

The purpose of this message is for the host to link n reports to an event(CEID). These linked event reports will default to ‘disabled’ upon linking. That is, the occurrence of an event would not cause the report to be sent until enabled. See S2F37 for enabling reports.

S2F36 Link Event Report Acknowledge  (E -> H)

Description:

Acknowledge or error if an error condition is detected the entire message is rejected(i.e., partial changes are not allowed).

S2F37 : Enable/Disable Event Report  (H -> E)

Description:

The purpose of this message is for the host to enable or disable reporting for a group of events(CEIDs).

S2F38 : Enable/Disable Event Report Acknowledge  (E -> H)

Description:

Acknowledge or error if an error condition is detected the entire message is rejected(i.e., partial changes are not allowed).

S2F41 : Remote Command Send  (H -> E)

Description:

The host requests the equipment perform the specified remote command with the associated parameters.

S2F42 : Remote Command Acknowledge  (E -> H)

Description:

Acknowledge host’s command or error. If command is not accepted due to one or more invalid parameters, then a list of invalid parameters will be returned containing the parameter name and reason for being invalid.

S5F1 : Alarm Report  (E -> H)

Description:

This message reports a change in or presence of an alarm condition. One message will be issued when the alam is set and one message will be issued when the alarm is cleared. Irrecoverable errors and attention flags may not have a corresponding clear message.

S5F2 : Alarm Acknowledge  (H -> E)

Description:

The host acknowledges the alarm report. This equipment ignores the ACKC5.

S5F3 Enable/Disable Alarm Send  (H -> E)

Description:

This message will change the state of the enable bit in the equipment. The enable bit determines if the alarm will be sent to the host. Alarms which are not controllable in this way are unaffected by this message.

S5F4 Enable/Disable Alarm Acknowledge  (E -> H)

Description:

The host acknowledges the alarm report. This equipment ignores the ACKC5.

S5F5 : List Alarms Request  (H -> E)

Description:

This message requests the equipment to send binary and analog alarm information to the host.

S5F6 : List Alarm Data  (E -> H)

Description:

This message contains the alarm data known to the equipment.

S5F7 : List Enabled Alarms Request  (H -> E)

Description:

List all alarms which are enabled .

S5F8 : List Enabled Alarm Data  (E -> H)

Description:

This message is similar to S5F6 except that it lists only alarms which are enabled.

S6F1 : Trace Data Send  (E -> H)

Description:

This function sends samples to the host according to the trace setup done by S2F23. Trace is a time-driven form of equipment status.

S6F2 : Trace Data Acknowledge  (H -> E)

Description:

List all alarms which are enabled .

S6F11 : Event Report Send  (E -> H)

Description:

The purpose of this message is for the equipment to send a defined, linked, and enabled group of reports to the host upon the occurrence of an event(CEID).

S6F12 : Event Report Acknowledge  (H -> E)

Description:

Acknowledge or error for Event Report Send(S6F11).

S6F15 : Event Report Request  (H -> E)

Description:

The purpose of this message is for the host to demand a given report group form the equipment.

S6F16 : Event Report Data  (E -> H)

Description:

EQ sends reports linked to given CEID to host.

S6F19 : Individual Report Request  (H -> E)

Description:

Host requests a Report for the specified RPTID.

S6F20 : Individual Report Data Event Report Data  (E -> H)

Description:

EQ sends variable data defined for the given to the host.

S7F1 : Process Program Load Inquire  (H -> E)

Description:

The sender requests permission to send a process program.

S7F2 : Process Program Load Grant  (E -> H)

Description:

The receiver grants permission to send a process program.

S7F5 : Process Program Request  (H -> E)

Description:

Request the other end of the link to send a specified process program from its library.

S7F6 : Process Program Data  (E -> H)

Description:

Send the requested process program.

S7F19 : Process Program Dir. Request  (H -> E)

Description:

The host requests a directory of the process program currently in the EQ’s library.

S7F20 : Current Process Program Data  (E -> H)

Description:

The EQ sends a directory of its process program library. If the library is empty, the list will be zero length.

S7F25 : Formatted Process Program Request  (H -> E)

Description:

The host requests a process program currently in the EQ’s library.

S7F26 : Formatted Process Program Data  (E -> H)

Description:

The EQ sends a formatted process program data to the host.

S9F7 : Invalid Data

Description:

This message indicates that the stream and function were recognized, but the associated data format could not be interpreted.

S9F9 : Transaction Timer Timeout

Description:

This message indicates that a transaction (receive) timer has timed out and that the corresponding transaction has been aborted. It is up to the host to respond to this error in an appropriate manner to keep the system operational.

S10F1 : Terminal Request

Description:

A terminal text message to the host.

S10F2 : Terminal Request Acknowledge

Description:

Acknowledge or error for terminal request (S10F1).

S10F3 : Terminal Display, Single  (H -> E)

Description:

Data to be displayed.

S10F4 : Terminal Request Acknowledge  (E -> H)

Description:

Acknowledge or error for terminal request (S10F3).

PPBODY – AOI

| ID | Name | Enable | Unit | Format | Min | Max | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 101 | AOI_CornerA_X | TRUE | mm | ASCII | 0 | 500 | Boat A角落的X位置 |
| 102 | AOI_CornerA_Y | TRUE | mm | ASCII | 0 | 500 | Boat A角落的Y位置 |
| 103 | AOI_CornerC_X | TRUE | mm | ASCII | 0 | 500 | Boat C角落的X位置 |
| 104 | AOI_CornerC_Y | TRUE | mm | ASCII | 0 | 500 | Boat C角落的Y位置 |
| 114 | AOI_Substrate_Row | TRUE | Number | ASCII | 1 | 2 | 產品參數 |
| 115 | AOI_Substrate_Col | TRUE | Number | ASCII | 1 | 1 | 產品參數 |
| 116 | AOI_Block_Row | TRUE | Number | ASCII | 1 | 4 | 產品參數 |
| 117 | AOI_Block_Col | TRUE | Number | ASCII | 1 | 4 | 產品參數 |
| 118 | AOI_Unit_Row | TRUE | Number | ASCII | 2 | 20 | 產品參數 |
| 119 | AOI_Unit_Col | TRUE | Number | ASCII | 2 | 20 | 產品參數 |
| 120 | AOI_Boat_Left_Offset | TRUE | pixel | ASCII | 0 | 3500 | Boat偏移量 |
| 121 | AOI_Boat_Right_Offset | TRUE | pixel | ASCII | 0 | 3500 | Boat偏移量 |
| 122 | AOI_Boat_Top_Offset | TRUE | pixel | ASCII | 0 | 2500 | Boat偏移量 |
| 123 | AOI_Boat_Bottom_Offset | TRUE | pixel | ASCII | 0 | 2500 | Boat偏移量 |

PPBODY - Sorter

| ID | Name | Enable | Unit | Format | Min | Max | Explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 201 | Sorter_Recipe_Name | TRUE | Null | A |  |  | Recipe名稱 |
| 202 | Sorter_Recipe_ProductName | TRUE | Null | A |  |  | 產品名稱 |
| 203 | Sorter_Recipe_X_Num | TRUE | Number | A | 1 | 20 | 橫向料數 |
| 204 | Sorter_Recipe_Y_Num | TRUE | Number | A | 1 | 10 | 縱向料數 |
| 205 | Sorter_Recipe_Board1_P1_X | TRUE | mm | A | 41000 | 42500 | NG區軌道P1點X |
| 206 | Sorter_Recipe_Board1_P1_Y | TRUE | mm | A | 551500 | 552500 | NG區軌道P1點Y |
| 207 | Sorter_Recipe_Board1_P1_Z | TRUE | mm | A | 73500 | 74500 | NG區軌道P1點Z |
| 208 | Sorter_Recipe_Board1_P4_X | TRUE | mm | A | 311000 | 312000 | NG區軌道P4點X |
| 209 | Sorter_Recipe_Board1_P4_Y | TRUE | mm | A | 672000 | 673000 | NG區軌道P4點Y |
| 210 | Sorter_Recipe_Board1_P4_Z | TRUE | mm | A | 73500 | 74500 | NG區軌道P4點Z |
| 211 | Sorter_Recipe_Board2_P1_X | TRUE | mm | A | 41000 | 42500 | OK區軌道P1點X |
| 212 | Sorter_Recipe_Board2_P1_Y | TRUE | mm | A | 325500 | 327500 | OK區軌道P1點Y |
| 213 | Sorter_Recipe_Board2_P1_Z | TRUE | mm | A | 73500 | 74500 | OK區軌道P1點Z |
| 214 | Sorter_Recipe_Board2_P4_X | TRUE | mm | A | 311000 | 312000 | OK區軌道P4點X |
| 215 | Sorter_Recipe_Board2_P4_Y | TRUE | mm | A | 445500 | 447500 | OK區軌道P4點Y |
| 216 | Sorter_Recipe_Board2_P4_Z | TRUE | mm | A | 73500 | 74500 | OK區軌道P4點Z |
| 217 | Sorter_Recipe_Board3_P1_X | TRUE | mm | A | 41000 | 42500 | ST區軌道P1點X |
| 218 | Sorter_Recipe_Board3_P1_Y | TRUE | mm | A | 99000 | 101000 | ST區軌道P1點Y |
| 219 | Sorter_Recipe_Board3_P1_Z | TRUE | mm | A | 73500 | 74500 | ST區軌道P1點Z |
| 220 | Sorter_Recipe_Board3_P4_X | TRUE | mm | A | 311000 | 313000 | ST區軌道P4點X |
| 221 | Sorter_Recipe_Board3_P4_Y | TRUE | mm | A | 220000 | 221000 | ST區軌道P4點Y |
| 222 | Sorter_Recipe_Board3_P4_Z | TRUE | mm | A | 73500 | 74500 | ST區軌道P4點Z |
| 223 | Sorter_Recipe_Zdown_Position | TRUE | mm | A | 80000 | 90000 | 下壓高度 |

MAGAZINE MOVEIN LOG

Description:

MagazineID attached to the MagazineMoveInEvent(101).

※Please return “PASS” or “NG” with MAGAZINE_MOVEIN_STATUS

BOAT MOVEIN LOG

Description:

BoatBarcode attached to the BoatMoveInEvent(102).

※Please return “PASS” or “NG” with BOAT_MOVEIN_STATUS

PASSBOAT MOVEOUT LOG

Description:

PassBoatBarcode attached to the PassBoatMoveOutEvent(103).

NGBOAT MOVEOUT LOG

Description:

NGBoatBarcode attached to the NGBoatMoveOutEvent(104).

EMPTYBOAT RELEASE LOG

Description:

EmptyBoatBarcode attached to the EmptyBoatReleaseEvent(105).

※Please return “PASS” or “NG” with EMPTYBOAT_RELEASE_STATUS

PASS1MAGAZINE MOVEIN LOG

Description:

Pass1MagazineID attached to the Pass1MagazineMoveInEvent(106).

※Please return “PASS” or “NG” with PASS1MAGAZINE_MOVEIN_STATUS

PASS2MAGAZINE MOVEIN LOG

Description:

Pass2MagazineID attached to the Pass2MagazineMoveInEvent(107).

※Please return “PASS” or “NG” with PASS2MAGAZINE_MOVEIN_STATUS

NGMAGAZINE MOVEIN LOG

Description:

NGMagazineID attached to the NGMagazineMoveInEvent(108).

※Please return “PASS” or “NG” with NGMAGAZINE_MOVEIN_STATUS

EMPTYMAGAZINE MOVEIN LOG

Description:

EmptyMagazineID attached to the EmptyMagazineMoveInEvent(109).

※Please return “PASS” or “NG” with EMPTYMAGAZINE_MOVEIN_STATUS

PASS1MAGAZINE MOVEOUT LOG

Description:

Pass1MagazineID attached to the Pass1MagazineMoveOutEvent(110).

※Please return “PASS” or “NG” with PASS1MAGAZINE_MOVEOUT_STATUS

PASS2MAGAZINE MOVEOUT LOG

Description:

Pass2MagazineID attached to the Pass2MagazineMoveOutEvent(111).

※Please return “PASS” or “NG” with PASS2MAGAZINE_MOVEOUT_STATUS

NGMAGAZINE MOVEOUT LOG

Description:

NGMagazineID attached to the NGMagazineMoveOutEvent(112).

※Please return “PASS” or “NG” with NGMAGAZINE_MOVEOUT_STATUS

BOAT ASSING NGBOAT LOG

Description:

BoatAssingNGBoatBarcode attached to the BoatAssingNGBoatEvent(113).

※Please return “PASS” or “NG” with BOAT_ASSING_NGBOAT_STATUS

UI LOCK COMMAND LOG

Description:

UI Lock state switched result attached to the RemoteActionResultEvent(114).

★Remote control UI lock log. (SUCCESS or FAIL)

★Remote control UI unlock log. (SUCCESS or FAIL)

EQUIPMENT OPERATIONAL COMMAND LOG

Description:

Equipment Operational state switched result attached to the RemoteActionResultEvent(114).

★Remote control equipment start log. (SUCCESS or FAIL)

★Remote control equipment stop log. (SUCCESS or FAIL)

EQUIPMENT ENDLOT COMMAND LOG

Description:

Equipment endlot result attached to the RemoteActionResultEvent(114).

EQUIPMENT STARTRUN CHECK LOG

Description:

RecipeName attached to the EquipmentStartRunEvent(115).

※Please return “PASS” or “NG” with EQUIPMENT_STARTRUN_STATUS